Part 3  Device IP Addresses

| Device | Interface | IPv6 Address | Purpose |
| --- | --- | --- | --- |
| R0 | GigabitEthernet0/0 | 2001:DB8:3:1::1/64 | Link to R1 |
| R0 | GigabitEthernet0/1 | 2001:DB8:1:1::1/64 | LAN for PC1 |
| R1 | GigabitEthernet0/0 | 2001:DB8:3:2::2/64 | Link to R2 |
| R1 | GigabitEthernet0/1 | 2001:DB8:3:1::2/64 | Link to R0 |
| R2 | GigabitEthernet0/0 | 2001:DB8:3:2::1/64 | Link to R1 |
| R2 | GigabitEthernet0/1 | 2001:DB8:1:2::1/64 | LAN for PC2 |
| PC1 | NIC | 2001:DB8:1:1:202:16FF:FE18:5893 | Host in R0’s LAN |
| PC2 | NIC | 2001:DB8:1:1:250:FFF:FE13:50DA | Host in R0’s LAN |
| PC3 | NIC | 2001:DB8:1:2:2D0:BCFF:FE63:D7BD | Host in R2’s LAN |
| PC4 | NIC | 2001:DB8:1:2:260:47FF:FED3:9282 | Host in R2’s LAN |

Static Routing

| Router | Destination Network | Next Hop Address | Purpose |
| --- | --- | --- | --- |
| R0 | 2001:DB8:1:2::/64 | 2001:DB8:3:1::2 | Route to R2’s LAN via R1 |
| R0 | 2001:DB8:3:2::/64 | 2001:DB8:3:1::2 | Route to R2’s interface via R1 |
| R1 | 2001:DB8:1:1::/64 | 2001:DB8:3:1::1 | Route to R0’s LAN via R0 |
| R1 | 2001:DB8:1:2::/64 | 2001:DB8:3:2::1 | Route to R2’s LAN via R2 |
| R2 | 2001:DB8:1:1::/64 | 2001:DB8:3:2::2 | Route to R0’s LAN via R1 |
| R2 | 2001:DB8:3:1::/64 | 2001:DB8:3:2::2 | Route to R0’s interface via R1 |

PART 1 IP TABLES

  - Mesh Topology

| Device | Interface | IPv4 Address | Subnet Mask | Connected To |
| --- | --- | --- | --- | --- |
| Switch0 | VLAN 1 | 196.240.0.10 | 255.255.255.0 | PC2 |
| PC2 | NIC | 196.240.0.1 | 255.255.255.0 | Switch0 |
| Switch1 | VLAN 1 | 196.240.0.11 | 255.255.255.0 | PC3 |
| Host | NIC | 196.240.0.2 | 255.255.255.0 | Switch1 |
| Switch2 | VLAN 1 | 196.240.0.12 | 255.255.255.0 | PC0 |
| PC0 | NIC | 196.240.0.5 | 255.255.255.0 | Switch2 |
| Switch3 | VLAN 1 | 196.240.0.13 | 255.255.255.0 | PC1 |
| PC1 | NIC | 196.240.0.4 | 255.255.255.0 | Switch3 |
| Switch4 | VLAN 1 | 196.240.0.14 | 255.255.255.0 | PC4 |
| PC4 | NIC | 196.240.0.3 | 255.255.255.0 | Switch4 |

1.2.  Extended Star Topology

| Device Name | Connected Switch | Interface | IPv4 Address | Subnet Mask |
| --- | --- | --- | --- | --- |
| PC1 | Switch1 | NIC | 143.198.10.1 | 255.255.255.0 |
| PC2 | Switch1 | NIC | 143.198.10.2 | 255.255.255.0 |
| PC3 | Switch1 | NIC | 143.198.10.3 | 255.255.255.0 |
| COM4 | Switch2 | NIC | 143.198.10.4 | 255.255.255.0 |
| Laptop1 | Switch2 | NIC | 143.198.10.5 | 255.255.255.0 |
| COM5 | Switch2 | NIC | 143.198.10.6 | 255.255.255.0 |
| COM6 | Switch5 | NIC | 143.198.11.1 | 255.255.255.0 |
| COM7 | Switch5 | NIC | 143.198.11.2 | 255.255.255.0 |
| COM8 | Switch5 | NIC | 143.198.11.3 | 255.255.255.0 |

  - Ring Topology

| Device | Connected Switch | IP Address |
| --- | --- | --- |
| PC0 | Switch0 | 192.164.1.1 |
| PC1 | Switch1 | 192.164.1.2 |
| PC2 | Switch2 | 192.164.1.3 |
| PC3 | Switch3 | 192.164.1.4 |
| PC4 | Switch4 | 192.164.1.5 |
| PC5 | Switch1 | 192.164.1.6 |
| PC6 | Switch2 | 192.164.1.7 |
| PC7 | Switch3 | 192.164.1.8 |

  - Bus Topology

| Device | Connected Switch | IP Address |
| --- | --- | --- |
| PC0 | Switch1 | 192.162.1.1 |
| PC1 | Switch2 | 192.162.1.2 |
| PC2 | Switch3 | 192.162.1.3 |
| PC3 | Switch4 | 192.162.1.4 |
| PC4 | Switch5 | 192.162.1.5 |

1.5 Hybrid IP Address Table

Router Interfaces

Interface	IP Address	VLAN	Description

Gi0/0.10	192.168.10.1/24	10	Management

Gi0/0.20	192.168.20.1/24	20	Ring

Gi0/0.30	192.168.30.1/24	30	Bus

Gi0/0.40	192.168.40.1/24	40	Mesh

Gi0/0.50	192.168.50.1/24	50	Star

Gi0/0.60	192.168.60.1/24	60	Extended Star

Server

Device	IPv4	IPv6	Gateway

Server0	192.168.10.10	2001:DB8:10::10/64	192.168.10.1

PCs

PC	VLAN 	IPv4	             IPv6	            Topology

PC0	20	192.168.20.10	2001:DB8:20::10/64	Ring

PC1	20	192.168.20.11	2001:DB8:20::11/64	Ring

PC2	20	192.168.20.12	2001:DB8:20::12/64	Ring

PC3	30	192.168.30.10	2001:DB8:30::10/64	Bus

PC4	30	192.168.30.11	2001:DB8:30::11/64	Bus

PC5	30	192.168.30.12	2001:DB8:30::12/64	Bus

PC6	40	192.168.40.10	2001:DB8:40::10/64	Mesh

PC7	40	192.168.40.11	2001:DB8:40::11/64	Mesh

PC8	40	192.168.40.12	2001:DB8:40::12/64	Mesh

PC9	50	192.168.50.10	2001:DB8:50::10/64	Star

PC10	50	192.168.50.11	2001:DB8:50::11/64	Star

PC11	50	192.168.50.12	2001:DB8:50::12/64	Star

PC12	50	192.168.50.13	2001:DB8:50::13/64	Star

PC13	60	192.168.60.10	2001:DB8:60::10/64	Extended Star

PC14	60	192.168.60.11	2001:DB8:60::11/64	Extended Star

PC15	60	192.168.60.12	2001:DB8:60::12/64	Extended Star

PC16	60	192.168.60.13	2001:DB8:60::13/64	Extended Star

Star Topology IP table

| Device | Connected Switch | IP Address |
| --- | --- | --- |
| PC0 | Switch0 | 192.168.0.1 |
| PC1 | Switch0 | 192.168.0.4 |
| PC2 | Switch0 | 192.168.0.3 |
| PC3 | Switch0 | 192.168.0.6 |
| PC4 | Switch0 | 192.168.0.9 |
| PC9 | Switch0 | 192.168.0.8 |